# LLM-assisted Multi-layer Scenario Safety Constraint Identification module

# Construction of the EOS-STPA model for lane-change scenarios

Based on the three-layer ICM/SRM/CSM model, the system-level losses and scenario-level hazard events are defined by experts, as listed in Tables 1 and 2, respectively.

Table 1

Definitions of losses for lane-change scenarios.

| Loss ID | Description | Interpretation |
| --- | --- | --- |
| L-1 | Personal injury | Personal injury or fatality |
| L-2 | Vehicle losses | Vehicle damage or loss |
| L-3 | Mission failed | Mission failure |

Table 2

System-level hazards for lane-change scenarios and their related losses.

| Hazard ID | Description | Related Losses |
| --- | --- | --- |
| H-1 | Collision with target-lane vehicle | L-1, L-2, L-3 |
| H-2 | Collision with obstacles | L-1, L-2, L-3 |
| H-3 | Driving out of lane or entering unintended lane | L-3 |
| H-4 | Rear-ended by vehicle in current/target lane | L-1, L-2, L-3 |
| H-5 | Collision with facilities | L-1, L-2, L-3 |
| H-6 | Collision with other vehicles (e.g., motorcycles, etc.) | L-1, L-2, L-3 |

Based on the predefined system-level losses and hazard events, an expert-led EOS-STPA analysis was conducted for autonomous lane-change scenarios. A set of scenario-specific UCAs and their corresponding SSCs were thereby identified, as summarized in Tables 3 and 4.

Table 3

Baseline UCAs identified by expert-led EOS-STPA analysis for lane-change scenarios (predicates and guidewords are italicized).

| UCA ID | Description | Related Hazards |
| --- | --- | --- |
| UCA-1 | Environment provides bad weather leading to obstructed vision. | H-1, H-2, H-3, H-4, H-5, H-6 |
| UCA-2 | Sensor feedback is unable to recognize obstacles in any context. | H-2 |
| UCA-3 | Road level provides complex geometric structures in any context. | H-1, H-2, H-3, H-4, H-5, H-6 |
| UCA-4 | Road level provides blurred lane boundaries in any context. | H-3, H-5 |
| UCA-5 | Traffic infrastructure provides a late red light when braking is required. | H-6 |
| UCA-6 | Traffic infrastructure does not provide a red light when braking is required. | H-6 |
| UCA-7 | Traffic infrastructure provides an early green light when braking should be maintained. | H-6 |
| UCA-8 | Traffic infrastructure provides guidance information in an incorrect order. | H-1, H-2, H-3, H-4, H-5, H-6 |
| UCA-9 | Traffic infrastructure does not provide guidance information when environment perception is unavailable or incorrect. | H-3, H-5, H-6 |
| UCA-10 | Traffic infrastructure does not provide facility information when environment perception is unavailable or incorrect. | H-2, H-3, H-5 |
| UCA-11 | Incorrect relative distance information to target-lane vehicles is detected. | H-1, H-3, H-4, H-6 |
| UCA-12 | No information of target-lane vehicles is detected. | H-1, H-3, H-4, H-6 |
| UCA-13 | Wrong information of target-lane vehicles is detected. | H-1, H-3, H-4, H-6 |
| UCA-14 | HV initiates steering for lane change when the turn signal is not activated. | H-1, H-3, H-4, H-6 |
| UCA-15 | Target-lane vehicles do not provide feedback when steering is initiated. | H-1, H-3, H-4, H-6 |
| UCA-16 | Target-lane vehicles provide incorrect feedback when steering is initiated. | H-1, H-3, H-4, H-6 |

Table 4

Baseline SSCs derived from expert-identified UCAs using EOS-STPA.

| SSC ID | Scenario Safety Constraint | Related UCA |
| --- | --- | --- |
| SSC-1 | Sensor feedback shall not be vision-obstructed or unable to recognize objects when adverse weather conditions are present. | UCA-1 |
| SSC-2 | Sensor feedback shall not be unable to recognize obstacles. | UCA-2 |
| SSC-3 | Sensor feedback shall not exceed cognitive limits when complex road geometries are present. | UCA-3 |
| SSC-4 | Sensor feedback shall not be unable to recognize lane boundaries when road boundaries are blurred. | UCA-4 |
| SSC-5 | Vehicle action shall be braking when a late red light is provided by traffic infrastructure. | UCA-5 |
| SSC-6 | Vehicle action shall be braking when a red light is provided by traffic infrastructure. | UCA-6 |
| SSC-7 | Vehicle action shall not be acceleration when an early green light is provided and braking should be maintained. | UCA-7 |
| SSC-8 | HV shall enter a safe state when guidance information is provided in an incorrect order. | UCA-8 |
| SSC-9 | Sensor feedback shall not be unavailable or incorrect when traffic infrastructure does not provide guidance information. | UCA-9 |
| SSC-10 | Sensor feedback shall not be unavailable or incorrect when traffic infrastructure does not provide facility information. | UCA-10 |
| SSC-11 | Sensor feedback shall not provide incorrect distance information when the turn signal is activated. | UCA-11 |
| SSC-12 | Sensor feedback shall not fail to detect target-lane vehicle information when the turn signal is activated. | UCA-12 |
| SSC-13 | Sensor feedback shall not provide incorrect target-lane vehicle information when the turn signal is activated. | UCA-13 |
| SSC-14 | Vehicle action shall not be steering when the turn signal is not activated. | UCA-14 |
| SSC-15 | Sensor feedback shall not fail to detect target-lane vehicles' responses when steering is initiated. | UCA-15 |
| SSC-16 | Sensor feedback shall not provide incorrect target-lane vehicles' responses when steering is initiated. | UCA-16 |

# LLM-assisted SSC identification process

Finally, the experts merge the LLM-generated SSCs with the baseline SSCs to form the final set of SSCs for lane-change scenarios. The retained results are summarized in Table 5.

Table 5

LLM candidate UCAs, expert screening results, and newly derived SSCs.

| UCA ID | LLM Candidate UCA | Related Hazards | Expert decision | Renumbered UCA ID | Derived SSC |
| --- | --- | --- | --- | --- | --- |
| UCA-17 | HV fails to maintain sufficient perception accuracy for target lane, obstacles, traffic participants and lane markings in any context. | [H-1][H-2][H-3][H-4][H-5][H-6] | Discard (too generic / overlapped) | – | – |
| UCA-18 | HV continues lane change when environment perception is degraded by adverse weather or low visibility. | [H-1][H-2][H-3][H-4][H-5][H-6] | Retain | UCA-17 | SSC-17 HV shall not initiate lane change when environment perception is degraded by adverse weather or low visibility. [UCA-17] |
| UCA-19 | HV does not detect obstacles in the target lane during lane change. | [H-1][H-2][H-6] | Discard (covered by baseline / redundant) | – | – |
| UCA-20 | Lane-change decision module authorizes lane change when the target lane is unsafe. | [H-1][H-2][H-3][H-4][H-5][H-6] | Retain | UCA-18 | SSC-18 Lane-change decision module shall not authorize lane change when the target lane is unsafe. [UCA-18] |
| UCA-21 | HV triggers emergency braking during lane change without ensuring rear and lateral safety gaps. | [H-1][H-3][H-4][H-6] | Retain | UCA-19 | SSC-19 HV shall ensure rear and lateral safety gaps before triggering emergency braking during lane change. [UCA-19] |
| UCA-22 | HV initiates lane change when traffic signals or road signs indicate lane change is prohibited or unsafe. | [H-1][H-5][H-6] | Discard (out of current focus / overlapped) | – | – |
| UCA-23 | HV bases lane-change decision on outdated, delayed or missing traffic infrastructure information. | [H-1][H-3][H-5][H-6] | Discard (covered by baseline infrastructure UCAs) | – | – |
| UCA-24 | HV changes lane without maintaining a safe gap to rear vehicles in the current or target lane. | [H-1][H-3][H-4][H-6] | Retain | UCA-20 | SSC-20 HV shall not initiate lane change when safe gaps to rear vehicles in the current or target lane are not maintained. [UCA-20] |
| UCA-25 | HV performs lane change when lane markings are not reliably detected. | [H-1][H-3][H-6] | Discard (redundant with baseline boundary/perception issues) | – | – |
| UCA-26 | HV continues lane change when sudden illumination changes cause unreliable perception of lane and obstacles. | [H-1][H-2][H-3][H-5][H-6] | Discard (covered by perception degradation family) | – | – |